In digital communication, these five processes work together to convert continuous analog signals (like your voice) into a stream of 1s and 0s that can be reliably transmitted and processed by computers.  Here is what they do and their specific roles:

- Sampling What it is: The process of taking instantaneous measurements (samples) of a continuous analog signal at regular time intervals. Role: It converts the signal from the continuous time domain into a discrete time domain. According to the Nyquist theorem, the sampling rate must be at least twice the highest frequency of the original signal to ensure it can be perfectly reconstructed later.

- Quantisation What it is: The process of mapping the sampled continuous values to the nearest predetermined discrete levels. Role: It rounds off each sample to the nearest available step. This creates an approximation of the actual signal, converting a continuous amplitude into a discrete amplitude.

- Encoding What it is: The process of representing each quantized value with a unique sequence of binary digits (bits), such as and . Role: It assigns a digital word to every single quantized level. This puts the data into the standard digital format required for electronic storage, processing, and transmission.

- Modulator What it is: A device/process that takes the digital data (bits) and alters the properties of a high-frequency analog signal (called a carrier signal). Role: Baseband(original signal) digital signals cannot travel long distances through the air or fiber optics. The modulator varies the carrier signal's amplitude, frequency, or phase to "carry" the digital data across a transmission medium.  (Bonus) Demodulator What it is: The exact counterpart to the modulator, located at the receiving end of the communication link. Role: It extracts the original baseband digital signal (the and ) from the modified carrier wave so it can be decoded back into the origin

Modulation is required in a communication system to reduce antenna size, avoid signal mixing, and increase communication range. It works by mixing a low-frequency message signal with a high-frequency carrier wave.

An antenna must be reduced in size because transmitting audio signals directly requires a physically impossible antenna that would be kilometers high. Modulation fixes this by converting low-frequency audio into high-frequency waves, which naturally require much smaller antennas.

The four primary types of modulation—focusing on the core continuous-wave and fundamental techniques—are Amplitude Modulation (AM), Frequency Modulation (FM), Phase Modulation (PM), and Pulse Modulation (or digital shift keying variants like Amplitude/Frequency/Phase Shift Keying).

Analog and Continuous-Wave Modulation

- Amplitude Modulation (AM): Changes the height (amplitude) of the carrier wave while keeping its frequency and phase the same. Used in AM radio broadcasting.

Allen

- Frequency Modulation (FM): Changes the speed (frequency) of the carrier wave based on the input signal while keeping the amplitude steady. Used in FM radio and TV sound.

- Phase Modulation (PM):

Changes the wave's starting point (phase) in response to the data signal. Often used in mobile communication systems

example of analog to analog transmission

An analog-to-analog conversion (also called analog modulation) is the process of changing a feature of a high-frequency carrier wave to match the changes in an analog data signal. A common real-life example is FM and AM radio broadcasting, where audio voice or music (a low-frequency analog signal) modulates a high-frequency radio carrier wave so it can travel through the air.

An example of analog-to-analog transmission is commercial FM radio broadcasting, where an analog voice or music signal modifies an analog radio frequency carrier wave for wireless transmission.

Analog-to-analog transmission is the process of mapping analog data onto an analog carrier signal to allow it to travel efficiently through a medium.

Common Examples

- FM and AM Radio: Your voice or music (analog sound waves) is converted by a microphone into an electrical analog signal, which then modulates a high-frequency analog radio wave.

- Cable TV (Traditional): Legacy cable television distribution networks sent video and audio signals as continuous analog waves through coaxial cables.

- Landline Telephones: Traditional POTS (Plain Old Telephone Service) takes the analog sound of your voice and sends it over copper wires as a continuously varying electrical current.

Modulation Methods Used

To achieve this transmission, the system changes one of three characteristics of the analog carrier wave:

- Amplitude Modulation (AM): The carrier wave's height (amplitude) changes based on the data signal.

- Frequency Modulation (FM): The carrier wave's speed of oscillation (frequency) changes based on the data signal.

- Phase Modulation (PM): The carrier wave's starting point or angle (phase) changes based on the data signal.

The primary use of an antenna is to convert electrical signals into electromagnetic radio waves for wireless transmission through the air, and vice versa for reception.

It acts as the essential bridge between the electronic circuits of a device and the open air.

The Two Primary Functions

- Transmission (Transmitter): It takes an oscillating electrical current from a device(like a phone or radio), accelerates the electrons, and launches them into space as wireless radio waves.

- Reception (Receiver): It intercepts passing radio waves in the air and converts them back into tiny electrical currents for a device to process.

- Common Uses in Daily Life

Smartphones & Cellular: Connects your phone to cellular towers for calls and 4G/5G mobile data. Wi-Fi Routers: Broadcasts and receives internet data signals to connect your laptop, phone, and smart home devices. Satellite Communication: Dishes on houses or satellites in space focus narrow beams of signals to transmit TV, GPS, and weather data over massive distances. Broadcasting: Large towers broadcast AM/FM radio and over-the-air television signals to millions of home and car receivers. Radar Systems: Emits radio waves and catches the reflections to track airplanes, predict weather, and assist self-driving cars.

READ advantages and disavantages pf AM, FM, PM