### Signals and Noise

#### 1.1 What is a Signal?

- A signal is any physical quantity (Voltage, current, time) that carries information.

- In communication systems, signals are used to carry information from one point to another.

#### 1.2 Types of Signals

- Analog Signal

  - Varies continuously with time.

  - Example: Voice, music, video signals.

  - Representation:
s(t)=Asin⁡(2πft+ϕ)s(t) = A \sin(2\pi f t + \phi)s(t)=Asin(2πft+ϕ)

- Digital Signal

  - Has discrete values (usually 0 and 1).

  - Example: Computer data, binary codes.

#### Noise ( Definition)

Noise is any unwanted electrical signal that distorts or interferes with the transmission of information.

- Thermal Noise (Johnson Noise):

  - Caused by random motion of electrons in conductors.

  - Formula:
N=kTBN = kTBN=kTB
where
kkk = Boltzmann’s constant (1.38 × 10⁻²³ J/K)
TTT = Temperature (Kelvin)
BBB = Bandwidth (Hz)

- Intermodulation Noise:

  - Caused when signals at different frequencies mix and produce new frequencies.

- Crosstalk:

  - Interference between adjacent channels (e.g., in telephone lines).

- Impulse Noise:

  - Caused by sudden disturbances (e.g., lightning, switching).

Sample means a small part or a quantity.

Sampling theorem:-  A continuous time signal(CTS) can be completely represented its sampled form and can be recovered back when sampling frequency (Fs) is greater than or equal twice of max frequency(Fm) of the CTS(message signal).

Fs ≥ 2Fm  where Fm = max. frequency of the signal.

Nyquist rate: The minimum sampling rate Fs=2Fm(tolerate condition) required to recover the original signal (message signal) is known as Nyquist rate.

If Fs < Fm , original signal cannot be recovered.

Nyquist interval, Ts= 1/F=1/2Fm. This is the signal overlapping condition.

| Nyquist Capacity | C=2B log2 M | -Max data rate (noiseless channel) |
| --- | --- | --- |

Where  C=The maximum bit rate measured in bits per second ((bps).

B (Bandwidth): The range of frequencies of the channel measured in Hertz (Hz).

M (Signal Levels): The number of discrete voltage or signal levels used to represent data.

## Shannon Capacity

#### 3.1 Introduction

- Given by Claude Shannon, it defines the maximum data rate that can be transmitted over a noisy channel without error.

For a channel of bandwidth B Hz and signal-to-noise ratio S/N,

C=Blog2(1+S/N)

where
C = Channel capacity (bps)
B = Bandwidth (Hz)
S/N = Signal-to-noise ratio (linear, not dB)

- Signals carry information; noise distorts it.

- Nyquist theorem sets limits for noiseless channels.

- Shannon theorem defines capacity for real-world noisy channels.

- Together, they form the foundation of digital communication theory.

Ques, Given B=5 KHz, SNR =61, Find C.  units kbps

C=5x1000xlog(1+61)=5000xlog62=5000x6 bits/s =30000bits/s=30kbps.