page 16 of Analog_Digital.pdf  diagrams.

Bitrate

measures actual data (bits) per second (bps), while baud rate measures signal changes (symbols/pulses) per second (baud), showing how often the physical signal changes, with bitrate often higher than baud rate when a single signal change carries multiple bits (e.g., 4-QAM, 8-PSK), making bitrate crucial for data efficiency and baud rate for physical channel capacity. The key formula is: Bitrate = Baud Rate × Bits per Symbol.

Bitrate (Data Rate)

- Definition: The number of actual binary digits (bits) transmitted per second, like 1000 bits/sec (1 kbps) or 1,000,000 bits/sec (1 Mbps).

- Focus: Computer processing efficiency and actual data volume.

- Units: bps (bits per second), kbps, Mbps, Gbps.

Baud Rate (Symbol Rate)

- Definition: The number of signal changes or "symbols" (pulses, voltage levels) sent per second.

- Focus: The physical transmission channel's speed and bandwidth usage.

- Units: Baud (bd) or Symbols per second (Sps).

Key Differences & Relationship

- 1 Bit = 1 Baud: If one signal change carries exactly one bit (e.g., simple On/Off keying), then bitrate equals baud rate (e.g., 9600 baud = 9600 bps).

- Multiple Bits per Baud: Modern modulation (like QAM) sends multiple bits per signal change, so Bitrate > Baud Rate (e.g., 1000 baud transmitting 4 bits per symbol yields 4000 bps).

- Analogy: Think of a train: Baud rate is the number of trains arriving/departing per hour, while bitrate is the total number of passengers (bits) carried by those trains.

Example

- A 1200 baud modem using 4-PSK (Phase-Shift Keying) sends 2 bits per signal (since

22=42 squared equals 4

22=4 states), resulting in a bitrate of 1200 baud * 2 bits/baud = 2400 bps.

Formula Sheet

Bits per signal element: n = log₂(L) where L is no. of signa levels

Bit rate: Rb = S log₂(L) where S is the baud rate

Baud rate: S = Rb / log₂(L)

Signal-element duration: Ts = 1/S

Number of levels: L = 2^(Rb/S)